Basic Filters

Filter One

Figure 1.4: Mystery filter one

Build the circuit shown in figure 1.4 on your breadboard. (The Rs = 50 is the internal resistance assumed for the function generator, you do not need to include that.)

Now set up the function generator to generate a 8VPPK signal, starting at 10Hz. Vary the frequency through the range shown in table below and fill out the rest of the data.

| Freq. (Hz) | Vin (V) | Vout (V) | Period (S) | Time Shift | Phase Shift |
| --- | --- | --- | --- | --- | --- |
| 10 | 8 | 8.6 | 100ms | 0 us | 0 deg |
| 30 | 8 | 8.6 | 33ms | 0 us | 0 deg |
| 100 | 8 | 8.6 | 10ms | 70 us | 2 deg |
| 300 | 8 | 8.6 | 3.33ms | 20 us | 3.7 deg |
| 1k | 8 | 8.4 | 998.16 us | 10.04 us | 4.5 deg |
| 3k | 8 | 8.4 V | 333.96 us | 8.90us | 9.59 deg |
| 10k | 8 | 6 V | 99.980 us | 8.84us | 31 deg |
| 30k | 8 | 2.9 V | 33.288 us | 5.71us | 61.71 deg |
| 100k | 8 | 980mV | 10.007 us | 2.196 us | 79 deg |
| 300k | 8 | 300mV | 3.322 us | 790 ns | 85.4 deg |

Now, use the oscilloscope to find the corner frequency. Remember, this is the frequency where the output voltage is 0.707 x Vin, also called the -3dB point. To get started, look at the data you just collected and find which two frequencies bound the 0.707 x Vin point you’re looking for. A bit of trial and error from there will quickly get you to the corner frequency. Record the phase shift and what type of filter it is based on your data below.

| -3dB frequency: | 55kHz to 70kHz | (1) |
| --- | --- | --- |
| Phase shift at -3dB frequency: | 73.16 deg | (2) |
| Filter type: | Low pass | (3) |

Now that you have the corner frequency and the table data, plot the magnitude of Vout and the phase response in the range from 10Hz to 300kHz. Use a logarithmic scale for the frequency axis. Make sure to label your axis clearly! Google Sheets or Microsoft Excel should be able to do this.

| Graph of voltage VS frequency: | (4) |
| --- | --- |
| Graph of phase shift VS frequency: | (5) |

Filter Two

Figure 1.5: Mystery Filter Two

Build the circuit shown in figure 1.5 on your breadboard. (The Rs = 50 is the internal resistance assumed for the function generator, you do not need to include that.)

Now set up the function generator to generate a 8VPPK signal, starting at 10Hz. Vary the frequency through the range shown in table below and fill out the rest of the data.

| Freq. (Hz) | Vin (V) | Vout (V) | Period (S) | Time Shift | Phase Shift |
| --- | --- | --- | --- | --- | --- |
| 10 | 8 | 1.09 | 100ms | 0 us | 0 deg |
| 30 | 8 | 1.09 | 33.34ms | -362us | -3.91 deg |
| 100 | 8 | 1.13 | 9.99ms | -414us | -14.92 deg |
| 300 | 8 | 1.45 | 3.332ms | -298.6us | -32.26 deg |
| 1k | 8 | 3.18 | 999.44us | -113.68us | -40.95 deg |
| 3k | 8 | 6.3 | 333.02us | -22.02us | -23.08 deg |
| 10k | 8 | 8.2 | 99.94us | -1.788us | -6.44 deg |
| 30k | 8 | 8.4 | 33.33us | -110ns | -1.19 deg |
| 100k | 8 | 8.4 | 10.01us | -15ns | -0.54 deg |
| 300k | 8 | 8.4 | 3.345us | 16ns | 0 deg |

Now, use the oscilloscope to find the corner frequency. Remember, this is the frequency where the output voltage is 0.707 x Vin, also called the -3dB point. To get started, look at the data you just collected and find which two frequencies bound the 0.707 x Vin point you’re looking for. A bit of trial and error from there will quickly get you to the corner frequency. Record the phase shift and what type of filter it is based on your data below.

| -3dB frequency: | 2.3kHz < Vin < 24kHz -> Vout = 5.65V | (6) |
| --- | --- | --- |
| Phase shift at -3dB frequency: | -26.71 deg | (7) |
| Filter type: | High pass | (8) |

Now that you have the corner frequency and the table data, plot the magnitude of Vout and the phase response in the range from 10Hz to 300kHz. Use a logarithmic scale for the frequency axis. Make sure to label your axis clearly! Google Sheets or Microsoft Excel should be able to do this.

| Graph of voltage VS frequency: | (9) |
| --- | --- |
| Graph of phase shift VS frequency: | (10) |

Filter Three

Figure 1.6: Mystery Filter Three

Build the circuit shown in figure 1.6. Connect the function generator at 8VPPK and run through the same measurements, filling out the table as you go.

| Freq. (Hz) | Vin (V) | Vout (V) | Period (S) | Time Shift | Phase Shift |
| --- | --- | --- | --- | --- | --- |
| 10 | 8 | 0 | 0 ms | 0 | 0 deg |
| 30 | 8 | 0 | 0ms | 0 us | 0 deg |
| 100 | 8 | 0mV | 0 | 0 | 0 |
| 300 | 8 | 161mV | 3.3350ms | -784.4 us | -84.84 |
| 1k | 8 | 540mV | 999.48us | -223.16us | -80.24 |
| 3k | 8 | 2.21V | 333.84us | -57.94us | -62.69 |
| 10k | 8 | 1.59V | 99.920us | 20.908us | 75.32 |
| 30k | 8 | 440mV | 33.414us | 8.218us | 88.71 |
| 100k | 8 | 121mV | 9.987us | 2.445us | 88.13 |
| 300k | 8 | 0 | 0 | 0 | 0 |

Now, use the oscilloscope to find the two corner frequencies. This circuit has two of them! You’ll have to find both points where the voltage goes above and then back below the 0.707 x Vin point. Start by figuring out approximately where those would be based on your data, and then using trial and error to find the two frequencies.

| 1st -3dB frequency: | 2kHz | (11) |
| --- | --- | --- |
| Phase shift at 1st -3dB frequency: | -76.49 deg | (12) |
| 2nd -3dB frequency: | 20kHz | (13) |
| Phase shift at 2nd -3dB frequency: | 83.67 deg | (14) |
| Filter type: | Band-pass | (15) |

Now that you have the corner frequency and the table data, plot the magnitude of Vout and the phase response in the range from 10Hz to 300kHz. Use a logarithmic scale for the frequency axis. Make sure to label your axis clearly! Google Sheets or Microsoft Excel should be able to do this.

| Graph of voltage VS frequency: | (16) |
| --- | --- |
| Graph of phase shift VS frequency: | (17) |

LTSpice:

Experimenting with the component values to determine how they affect the cutoff frequency.

| Figure 1.7: Filter one | Figure 1.8: Filter two |
| --- | --- |

- Open a new LTSpice schematic, and build the two circuits above. Don’t forget to add a ground node at the negative terminal of the voltage supply!

|  | - Instead of entering 8VPP into the AC circuit, change the value of the voltage source to “AC 1” |
| --- | --- |
|  | AC Analysis tells LTSpice to sweep through a range of frequencies, in this case from 10Hz to 300kHz. Setting the type to decade tells LTSpice to use logarithmic for the output graphs. |

- Once finished, run the simulation and probe the Vout node for each circuit separately. You should have a graph that matches one of the example graphs at the beginning of the lab document. Screenshot both of these graphs.

|  | This is what the graph for filter 3 looks like when simulated on LTSpice. We aren’t asking you to simulate this one - so this image is just an example of the type of graph you should expect. The graphs for filters 1 and 2 shouldn’t have two corner frequencies like this one does. |
| --- | --- |

- Now, change the values for one component in each circuit. You can change the resistor, or the reactive component. The value you change it to doesn’t matter much, but keep it within 1/10th to 10x the original value.

- Rerun the simulation and plot the Vout graphs for each circuit separately. Screenshot these both.

Original

Modified values: Resistors to 30k

Write a paragraph explaining the differences you noticed in the simulations with the modified values. How did the cutoff frequency change? Did the output magnitude change at all? What else did you notice?

The magnitude nealy doubled with my modification. The peak of the magnitude takes place at the same frequency, yet the change seen in the modified circuits is that the inductive filter 2 has a much higher and sharper cutoff frequency; with a very steep increase in magnitude.

Study Questions:

- If you were to switch the order of the inductor and capacitor for filter 3, do you think it would still work the same? Explain your thinking, with drawings if necessary.

(Hint: Filter 3 is sort of a combination of the first two filters. How does the capacitor in filter 1 respond to frequencies above or below the cutoff frequency? What about the inductor in filter 2?)

No. The inductor would be used at the low impedance at low frequencies thus it would act as a low-pass filter as opposed to the high-pass it once was .

- You can make a low-pass filter with an inductor, or a high-pass with a capacitor. Draw the circuit diagrams for these and explain (at a high level) how they work as high and low pass filters.

RL lowpass:

Now we have an inductor in series with a resistor. We would have low frequencies at low impedance, as well as higher impedance when frequencies are high

RC highpass:The capacitor is now in series with the resistor. Now we would have a high impedance blocking low frequencies and low impedance at high frequencies. Acting as a high pass filter.

© 2024 Oregon State University